Ensuring Compliance with IEC 62443-4-2 CR4.3 Use of Security guidelines in Ubuntu 22.04

IEC 62443-4-2 Voc

Ubuntu

CONTENTS

Security functionalities:	2

Default open Ports	2

Basic System Services:	2

Network-related Services:	3

Security-related Services:	3

Device and Driver Management	3

CR 3.3 Security Features from:	4

integrity check of software/information/configuration/others	4

Strength of password-based authentication	4

Authenticator feedback	5

Unsuccessful login attempts	6

System use notification	6

Authorization enforcement, Permission mapping to roles	7

Security functionalities:

- Support of TLS v1.2 and TLS v1.3:

Ubuntu 22.04 supports TLS v1.2 and TLS v1.3 protocols, which are essential for establishing secure communication channels. These protocols provide robust encryption, ensuring that data transmitted over the network is protected against eavesdropping, tampering, and forgery.

- Signed Firmware Updates:

Ubuntu 22.04 ensures the integrity and authenticity of firmware updates through the use of signed updates. This feature guarantees that only firmware from trusted sources is installed, protecting the system from potential security threats posed by unauthorized or malicious updates.

- TLS Version Management:

Disabling of TLS v1.0 and TLS v1.1: Ubuntu 22.04 disables older versions of the TLS protocol (TLS v1.0 and TLS v1.1) to eliminate security risks associated with them. By enforcing the use of TLS v1.2 and TLS v1.3, Ubuntu ensures that communications are secured with the latest and most secure cryptographic standards.

Default open Ports

- SSH (TCP: 22): SSH provides a secure method for remote access and management of the system, ensuring that all data transferred is encrypted.

Basic System Services:

- Event Logging: The system logs events, which are crucial for monitoring, auditing, and troubleshooting, providing transparency and accountability.

- System Updates: Manages automatic updates via APT and unattended-upgrades, ensuring the system remains patched and secure.

- File Management Interface: Provides a secure and user-friendly interface for managing files, with restrictions in place to prevent unauthorized access.

Network-related Services:

- Network Profile Management: Ubuntu manages network configurations securely, ensuring that connections are established and maintained within trusted parameters.

- DHCP Client: Obtains IP addresses automatically, ensuring secure and consistent network connectivity.

- DNS Client: Resolves domain names, with DNS security features ensuring that domain name resolutions are protected against attacks.

- Firewall (UFW): The Uncomplicated Firewall (UFW) in Ubuntu provides network firewall services, essential for blocking unauthorized access and protecting the system from external threats.

Security-related Services:

- Antivirus (ClamAV or Equivalent): Provides real-time protection against malware and other security threats, ensuring that the system is safeguarded from known vulnerabilities.

- Security Monitoring (Equivalent to Windows Security Center): Continuously monitors and reports the system's security status, ensuring compliance with security policies.

Device and Driver Management

- Plug and Play: Automatically manages hardware devices, ensuring that only secure and compatible devices are enabled.

- Device Install Service: Securely handles the installation of device drivers, ensuring that only trusted and verified drivers are used.

CR 3.3 Security Features from:

- Trusted Platform Module (TPM): Ubuntu 22.04 can leverage TPM to securely store cryptographic keys, enhancing device authentication and data protection.

- Secure Boot: Ubuntu’s Secure Boot feature ensures that the system boots only with trusted software, preventing unauthorized or malicious software from being executed during the boot process.

- System Updates (APT, unattended-upgrades): Regularly updates the system with the latest security patches, reducing vulnerability exposure.

- Firewall (UFW): Configurable firewall settings that prevent unauthorized network access, ensuring compliance with security requirements.

- Disk Encryption (LUKS): Full-disk encryption via LUKS protects sensitive data by ensuring that the entire disk is encrypted, preventing unauthorized access even if the physical media is compromised.

- USB Protection: Ubuntu can be configured to restrict or block USB storage devices, minimizing the risk of malware introduction through removable media.

- External Device Management: Monitors and controls external device access, ensuring that only approved devices are connected to the system, in line with security policies.

- Secure Firmware Updates (NIST SP 800-147): Ubuntu ensures that firmware updates are securely delivered and applied, preventing unauthorized modifications and ensuring the integrity of the system.

integrity check of software/information/configuration/others

Password Uniqueness Requirement

To prevent credential reuse as required by IEC 62443, Debian 12 implements a password history policy via the pam_pwhistory module. By configuring the remember parameter in /etc/pam.d/common-password, the system maintains a secure history of previous password hashes and rejects any new password that matches the last 5 used entries. This mitigates the risk of credential compromise and ensures that every password change results in a unique, non-redundant credential.

Strength of password-based authentication

The system enforces password strength requirements in accordance with IEC 62443-4-2. Debian 12 utilizes the PAM (Pluggable Authentication Modules) framework to manage password complexity and aging policies. Specifically, the pam_pwquality module is configured in /etc/pam.d/common-password to enforce a minimum length of 12 characters, including uppercase, lowercase, digits, and special characters. Password expiration is managed via the chage utility (e.g., sudo chage -M 90 <username>) to ensure compliance with NIST SP 800-63B guidelines.

Authenticator feedback

The Debian 12 system ensures that authenticator feedback is obscured during the authentication process in compliance with IEC 62443-4-2 requirements. On the graphical login screen (LXDM), password characters are masked to prevent unauthorized visual observation, while the command-line interface (CLI/SSH) enforces a "no feedback" policy (no asterisks) by default for maximum security. Additionally, the system provides only generic failure messages for incorrect login attempts to mitigate account enumeration risks, ensuring no specific reasons are disclosed. These settings are centrally managed via PAM (Pluggable Authentication Modules) and /etc/login.defs, with pam_faillock (replacing the deprecated pam_tally2) utilized to monitor unsuccessful attempts while maintaining obscured feedback to unauthenticated users.

Unsuccessful login attempts

Debian 12 enforces account lockout policies using the PAM (Pluggable Authentication Modules) framework to limit consecutive unsuccessful login attempts, fulfilling the requirements of IEC 62443-4-2. System administrators can configure granular lockout behaviors—such as the maximum number of failed attempts and the duration of the lockout—by modifying /etc/security/faillock.conf or adjusting the pam_faillock.so module parameters within the PAM configuration files. While standard installations may not enforce a lockout by default, the system is fully capable of being hardened to lock an account after a specified threshold (e.g., 3 or 5 consecutive failed attempts) within a defined time window. These lockout events can be configured to reset automatically after a cooling-off period or require manual administrative intervention, effectively mitigating brute-force and credential-stuffing attacks across all system interfaces, including the LXDM graphical login and SSH remote sessions.

System use notification

The system provides the capability to display a system use notification message before graphical authentication, ensuring compliance with IEC 62443 requirements. Debian 12 offers this feature via configuration files and settings, which can be modified by users with administrator privileges. To show a message before the terminal login screen, you can configure the /etc/issue file to display a system message. However, for graphical login, the most effective method is using the "LXDM" (Lightweight X11 Display Manager) to display a custom welcome message. The message is displayed on the graphical login screen before the user is asked to enter credentials.

Authorization enforcement, Permission mapping to roles

To comply with authorization enforcement and role-to-permission mapping requirements under standards such as IEC 62443-4-2, Ubuntu 22.04 systems should define user roles and control access using Linux’s built-in user and group management mechanisms.

Defined Roles and Their Privileges:

Admins

Users with full administrative rights via the sudo group. They are responsible for system configuration, software installation, security enforcement, and access to all resources.

Regular Users

Standard users with no administrative privileges. They are restricted to performing routine tasks and using authorized applications. They cannot make system-level changes or access sensitive configurations.

Defined Groups for Access Control:

sudo

Grants full administrative privileges to members. Equivalent to the “Administrators” group on Windows.

users

Default group for all non-administrative users. Provides access to general system functions without elevated permissions.

Enforcement Mechanisms:

Access control is implemented using group membership, file system permissions, Access Control Lists(ACLs), Role-Based Access Control(RBAC)